MATERIAL SAFTEY DATA SHEET

- Commercial Product Name   : COPPER PHTHALOCYANINE BLUE

CRUDE

Company                              : Kesar Petroproducts  LTD.

404 Naman Centre,4th Floor,Plot No C-31,             			                            Bandra Kurla Complex ,Bandra (E) Mumbai-     
					    400051 
                                                        Tel. 022 -62422626

- Composition / Chemical  Characterization :

Product Identification      :  C..P.C Crude

C.I. No.                          :  74160

CAS No.	                 :  147-14-8

EINECS No.                   :  2056851

TSCA No.                       :  147-14-8

- Hazard Identification                :   Not Known

- First aid measure  :

Inhalation         :     If inhaled, move to fresh air. Aid in breathing if

necessary, and get medical attention.

Eyes                :      Flush eye with flowing water. If irritation develops,

consult an Ophthalmologist.

Skin                 :      Wash affected skin with soap and water.

Ingestion         :      If swallowed, dilute with water and induce vomiting.

Get immediate medical attention.

- Fire fighting measures :

  - Suitable extinguishing media       : Water spray, Foam.

  - Unsuitable extinguishing media   : Carbon  di-oxide

  - Gases of CO, CO2, NOX and other hazardous oxides could be generated.

  - Self- contained breathing apparatus for fighters.

- Accidental release measures       :

  - Personal precautions         : Clean up with the appropriate personal

equipment (see Section 8.)

  - Environmental precautions : Scoop or Sweep & transfer into a

closed container. Keep waste out of

sewers .

  - Method for cleaning up       : Scoop or Sweep & into a

container for reuse or disposal.

- Handling and storage  :

A moderately dry, well- ventilated area is considered adequate for

handling and storage. Use proper personal equipment (see Section8).

Direct sources of heat must be avoided.

- Exposure Control / personal protection :

  - Respiratory protection          : Local exhaust ventilation, Air-purifying

respirator.

  - Hand protection                    : Gloves

  - Eye protection                      : Goggles

  - Skin protection                     : Impervious clothing.

- Physical & Chemical properties :

Appearance: Blue Powder                        Flammability: None

Odor              : None                                  Auto Flammability: None

pH: 6-8 (at 10 gr/250ml H20)                     Explosive properties: None

Boiling point: Not applicable                     Oxidizing properties: None

Melting point            : Not applicable         Vapor pressure: Not applicable

Flash point               : Not applicable         Relative density        : 1.97

Solubility-water        : Insoluble in water    Solubility – fat           : #

Partition coefficient (n-Octanol / water): No data available.

Other Data               : None                 # will report later

- Stability and reactivity  : This product is stable and hazardous

polymerization will not occur.

- Toxicological information : Acute Oral Toxicity : LD50(rat)>5,000mg/kg

- Ecological information:      No data available.

- Disposal considerations :

Consult local expert for advice on disposal of this material. Ensure that disposal is in compliance with local, national regulations.

- Transport Information: GGV IMDG CODE: None; UN No. : None.

ICAO/IATA- DGR: None; RID/ ADR: None.

- Regulatory Information: Not applicable.

- Other Information :

We have described our products concerning possible safety requirements by the above mentioned information given to the best of our knowledge and experience. All data given are never meant to guarantee any quality descriptions nor product properties.